CCNA

Troubleshooting & Command Reference

TCP/IP  |  OSI  |  Cisco IOS  |  Windows  |  Flashcards

Egor Grebenuk ¦ Full-Stack & Network Engineer CCNA Certified → CCNP ENCORE (next)

Layers Covered:

🏢 L1 Physical  |  🔗 L2 Data Link (ARP, VLAN, STP, EtherChannel, CDP, Port Security)

🌐 L3 Network (IPv4, Routing, OSPF, DHCP, NAT, ICMP)  |  🚚 L4 Transport (TCP/UDP)

🔍 L7 Application (DNS, SSH)  |  📐 OSI/TCP-IP Quick Reference

# 📐  OSI & TCP/IP Model — Quick Reference

Every troubleshooting session starts here. Map symptoms to the right layer before touching a single command.

| # | OSI LAYER | TCP/IP LAYER | KEY PROTOCOLS | START HERE IF... |
| --- | --- | --- | --- | --- |
| 7 | Application | Application | HTTP, DNS, SMTP, SSH, FTP | App errors, DNS fails, login issues |
| 6 | Presentation | Application | TLS/SSL, encoding | Cert errors, encoding issues |
| 5 | Session | Application | NetBIOS, RPC, SMB | Auth fails, session drops |
| 4 | Transport | Transport | TCP, UDP | Port blocked, retransmits, slow transfer |
| 3 | Network | Internet | IP, ICMP, OSPF, NAT | No route, ping fails, wrong IP |
| 2 | Data Link | Network Access | Ethernet, ARP, STP, VLAN | Wrong VLAN, MAC issues, loop |
| 1 | Physical | Network Access | Cables, SFP, NIC, PoE | No link, CRC errors, flapping |

## 🧠 Troubleshooting Philosophy — Bottom-Up

Always start at Layer 1 and move up. Don't skip layers.

1.  L1 Physical — Is cable connected? Link LED on? SFP ok?

2.  L2 Data Link — Is port up? Correct VLAN? No err-disabled?

3.  L3 Network — Is IP correct? Gateway reachable? Route exists?

4.  L4 Transport — Is port open? TCP handshake completing?

5.  L7 Application — DNS resolving? Service responding?

## ⚡ OSI Flashcards

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| No link light on switch port | show interfaces status | See connected/notconnect/err-disabled |
| CRC errors on port | show interfaces counters errors | Bad cable or duplex mismatch |
| Ping works, app fails | Check L4/L7 — not a routing issue | Split OSI layers in your head |
| Same subnet, can't reach device | arp -a / show arp | ARP = L2→L3 bridge |
| Ping works but slow | tracert / traceroute | Find the slow hop — L3 routing issue |
| Can reach gateway, not internet | show ip route / ipconfig | Default route missing at L3 |

# 🏢  Layer 1 — Physical

Responsible for physical connectivity. Check here first: no link = everything below is irrelevant.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show interfaces | Speed, duplex, CRC errors, input/output errors per interface |
| show interfaces status | Port state: connected ¦ notconnect ¦ err-disabled ¦ disabled |
| show interfaces counters errors | CRC, runts, giants, input errors — bad cable indicator |
| show interfaces transceiver | SFP optical signal levels (Tx/Rx power) |
| show power inline | PoE budget and per-port power delivery status |
| show inventory | Installed modules and SFP transceivers |
| show logging | Link up/down events with timestamps |
| show interfaces g0/1 ¦ inc CRC¦error¦rate | Filter only error counters for a specific port |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| Get-NetAdapter | Adapter status Up/Down, speed, name |
| Get-NetAdapter ¦ ft Name,Status,LinkSpeed | Confirmed link speed (100M/1G/10G) |
| Get-NetAdapterStatistics | Interface errors and dropped packets |
| Get-NetAdapterAdvancedProperty | Speed & Duplex settings |
| Get-PnpDevice -Class Net | NIC driver health |
| Disable-NetAdapter -Name 'Ethernet' | Bounce (disable) adapter |
| Enable-NetAdapter -Name 'Ethernet' | Re-enable adapter |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| No link light on port | show interfaces status | Confirms physical connection state |
| Intermittent disconnects | show interfaces g0/1 ¦ inc CRC | CRC = bad cable or duplex mismatch |
| Speed negotiated at 100M not 1G | show interfaces g0/1 | Check speed/duplex line |
| IP Phone has no power | show power inline | PoE budget or port config issue |
| SFP link not coming up | show interfaces transceiver | Check optical Tx/Rx power levels |
| NIC shows disconnected in Windows | Get-NetAdapter | Verify adapter status |
| Want to bounce NIC driver | Disable/Enable-NetAdapter | Restarts driver without reboot |
| Check for CRC on all ports | show interfaces counters errors | Scan all ports at once |

## 🔄 Troubleshooting Workflow

1.  Check Windows adapter → Get-NetAdapter

2.  Check switch port status → show interfaces status

3.  Check for CRC/errors → show interfaces counters errors

4.  If errors high → replace patch cable, test different port

5.  If still failing → check SFP with show interfaces transceiver

6.  Check driver → Get-PnpDevice -Class Net

7.  PoE device dead → show power inline

## ✅ Best Practices

- Always use Auto Negotiation on access ports

- Label all cables and patch panel ports

- Clean fiber optic connectors before inserting SFP

- Disable unused ports: shutdown

- Check PoE budget before adding new IP phones/APs

- Investigate CRC > 0 — never ignore physical errors

# 🔗  Layer 2 — ARP (Address Resolution Protocol)

ARP maps IP→MAC inside the local subnet. Without it, even same-subnet communication fails.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show arp | Full ARP table on router/L3 switch |
| show ip arp | IP↔MAC mappings with interface and age |
| show ip arp 192.168.1.10 | ARP entry for a specific IP |
| clear arp-cache | Force fresh ARP resolution for all entries |
| show logging ¦ include ARP | ARP-related log events |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| arp -a | View full ARP table (IP ↔ MAC) |
| arp -d * | Flush entire ARP cache |
| arp -d 192.168.1.50 | Delete one ARP entry |
| Get-NetNeighbor | PowerShell ARP table view |
| Remove-NetNeighbor -IPAddress 192.168.1.50 | Delete entry via PowerShell |
| ping 192.168.1.1 | Forces new ARP request for target |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Cannot reach printer after replacement | arp -d 192.168.1.50 → ping | Old MAC cached — force new ARP |
| Intermittent gateway unreachable | arp -a → check gateway MAC | Duplicate IP or ARP spoof |
| Random users lose access intermittently | arp -a → look for duplicate MACs | IP conflict = two MACs for one IP |
| Server migrated — clients cant reach it | arp -d * on clients | Old MAC still cached on all clients |
| Check ARP on router | show ip arp | Verify correct IP↔MAC mapping |

## 🔄 Troubleshooting Workflow

1.  Get device IP → ipconfig /all

2.  Check ARP table → arp -a

3.  Compare ARP MAC with actual device MAC

4.  If mismatch → arp -d <IP> then ping <IP>

5.  Check ARP again → arp -a

6.  If MAC keeps changing → check for duplicate IP or ARP spoof

7.  Locate device on switch → show mac address-table | include <MAC>

## ✅ Best Practices

- Clear ARP cache after replacing printer, server, or switch

- Use DHCP reservations for printers and servers (stable IP = stable ARP)

- Enable Dynamic ARP Inspection (DAI) in production

- Investigate any IP that appears with multiple MACs immediately

- Monitor syslog for duplicate IP/MAC warnings

# 🏷️  Layer 2 — VLANs & Trunking

VLANs segment broadcast domains. Trunks carry multiple VLANs between switches using 802.1Q tagging.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show vlan brief | All VLANs and their assigned ports at a glance |
| show interfaces trunk | Active trunks, encapsulation, allowed VLANs, native VLAN |
| show interfaces g0/1 switchport | Port mode (access/trunk), access VLAN, voice VLAN |
| show running-config interface g0/1 | Exact port config from running config |
| show spanning-tree vlan 10 | STP state for a specific VLAN |
| show mac address-table vlan 10 | All MACs learned in a VLAN |
| show logging ¦ include VLAN | VLAN-related events and mismatch warnings |
| show interfaces status | All ports: state, VLAN, speed, duplex |
| show interfaces g0/1 trunk | Trunk details for one specific port |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| User gets wrong IP address | show interfaces g0/1 switchport | Wrong access VLAN → wrong DHCP scope |
| New department has no network | show interfaces trunk | VLAN not in allowed list on trunk |
| Remote switch users offline | show interfaces trunk | VLAN not passing across trunk |
| IP Phone not registering | show interfaces g0/1 switchport | Voice VLAN missing or wrong |
| Native VLAN mismatch warning in logs | show logging ¦ inc VLAN | Different native VLANs on trunk ends |
| VLAN exists but traffic not passing | show spanning-tree vlan 10 | VLAN blocked by STP |
| Which ports are in VLAN 20? | show vlan brief | Instant VLAN membership view |
| Port showing as trunk when should be access | show interfaces g0/1 switchport | DTP auto-negotiated trunk — lock it |

## 🔄 Troubleshooting Workflow

1.  Find user's switch port → show interfaces status

2.  Check port mode and VLAN → show interfaces g0/1 switchport

3.  Verify VLAN exists → show vlan brief

4.  Check trunk carries the VLAN → show interfaces trunk

5.  If VLAN missing on trunk → switchport trunk allowed vlan add <ID>

6.  Verify native VLAN matches on both sides

7.  Check STP for the VLAN → show spanning-tree vlan <ID>

8.  Verify correct DHCP scope is assigned → check client IP

## ✅ Best Practices

- Never use VLAN 1 for users or management

- Use dedicated management VLAN (e.g. VLAN 99)

- Use dedicated voice VLAN (e.g. VLAN 100)

- Allow only required VLANs on trunk — never trunk all

- Set native VLAN to an unused VLAN on all trunks

- Disable DTP: switchport nonegotiate

- Document VLAN names, subnets, and purpose

- Monitor native VLAN mismatch events in syslog

# 🌳  Layer 2 — STP (Spanning Tree Protocol)

STP prevents Layer 2 loops. Without it: broadcast storm → MAC flapping → complete network failure.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show spanning-tree | Root bridge, port roles (Root/Designated/Blocked), port states |
| show spanning-tree root | Root bridge ID, priority, and cost for all VLANs |
| show spanning-tree vlan 10 | STP state for specific VLAN |
| show spanning-tree detail | Topology change count and timestamp — find loop source |
| show spanning-tree detail ¦ include occurr¦from | Filter for TC source port |
| show spanning-tree interface g0/1 | Role and state of one specific port |
| show spanning-tree blockedports | All ports currently blocked by STP |
| show spanning-tree summary | STP mode, root ports, blocked ports overview |
| show spanning-tree inconsistentports | Ports blocked by BPDU Guard or Loop Guard |
| show processes cpu sorted | High CPU during broadcast storm |
| show logging ¦ include STP¦TOPOLOGY | STP events and topology changes |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Entire floor lost network suddenly | show processes cpu sorted | Broadcast storm = CPU spike |
| Users disconnect every 30 seconds | show spanning-tree detail | Frequent topology changes = STP reconvergence |
| DHCP takes 30+ seconds on new connection | show spanning-tree interface g0/1 | PortFast missing on access port |
| New switch installation broke network | show spanning-tree root | New switch may have won Root election |
| MAC flapping messages in logs | show logging ¦ include MAC | Layer 2 loop present |
| Which ports are blocked? | show spanning-tree blockedports | Instant blocked port view |
| Find port causing topology changes | show spanning-tree detail ¦ include occurr¦from | Identifies TC source |
| Unknown switch added by user | show cdp neighbors | Find unauthorized device, enable BPDU Guard |

## 🔄 Troubleshooting Workflow

1.  Verify Root Bridge is the correct switch → show spanning-tree root

2.  Check topology change count → show spanning-tree detail

3.  Find the TC source port → show spanning-tree detail | include occurr|from

4.  Inspect blocked ports → show spanning-tree blockedports

5.  If broadcast storm suspected → show processes cpu sorted

6.  Check for MAC flapping → show logging | include MAC

7.  Find unauthorized switch → show cdp neighbors

8.  Isolate offending port → shutdown the port

9.  Restore Root Bridge priority if wrong → spanning-tree vlan X priority 4096

## ✅ Best Practices

- Set explicit Root Bridge: spanning-tree vlan X priority 4096

- Set explicit Secondary Root: spanning-tree vlan X priority 8192

- Enable PortFast on ALL access ports (never on trunk)

- Enable BPDU Guard globally: spanning-tree portfast bpduguard default

- Never connect unmanaged switches without approval

- Monitor topology change counters weekly

- Investigate MAC flapping immediately — it means a loop

- Document intended Root Bridge placement

# 🔀  Layer 2 — EtherChannel (LACP / PAgP)

EtherChannel bundles multiple physical links into one logical link. Increases bandwidth and provides redundancy without STP blocking.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show etherchannel summary | All port-channels: protocol, member ports, state (SU=up, SD=down) |
| show etherchannel detail | Detailed per-port info including LACP PDU counters |
| show etherchannel port-channel | Port-channel interface status |
| show interfaces port-channel 1 | Bandwidth, errors on the logical bundle |
| show lacp neighbor | Verify LACP negotiation with the remote switch |
| show lacp counters | LACP PDU send/receive counts per port |
| show pagp neighbor | PAgP neighbor info (Cisco proprietary) |
| show running-config interface g0/1 | Physical port channel membership config |
| show spanning-tree ¦ include Po | Verify STP sees port-channel, not individual ports |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| EtherChannel not forming | show etherchannel summary | Look for (I) individual — mismatch in config |
| One port not joining bundle | show lacp neighbor | Speed/duplex/VLAN mismatch on that port |
| Port-channel flapping | show interfaces port-channel 1 | Check errors and member port health |
| LACP neighbor not seen | show lacp neighbor | Wrong mode: use active/active or active/passive |
| STP still blocking one physical port | show spanning-tree ¦ include Po | STP should see Po1, not individual ports |
| Low bandwidth on bundle | show interfaces port-channel 1 | Load balance hash may not distribute evenly |
| After config change, channel broke | show etherchannel summary | Check all ports in (P) state, not (I) |

## 🔄 Troubleshooting Workflow

1.  Check bundle state → show etherchannel summary

2.  Verify LACP negotiation → show lacp neighbor

3.  All member ports must match: speed, duplex, VLAN config, native VLAN

4.  Check protocol: use LACP (open standard) not PAgP (Cisco only) for cross-vendor

5.  Verify STP sees the bundle as one port → show spanning-tree | include Po

6.  If a port shows (I) individual → fix the mismatched config on that port

## ✅ Best Practices

- Prefer LACP over PAgP (works with non-Cisco switches)

- Use active/active or active/passive — never passive/passive

- All member ports must have identical: speed, duplex, VLAN, native VLAN, STP settings

- Do not run STP on individual member ports — STP must see port-channel only

- Document port-channel number and member ports

- Use port-channel load balance: port-channel load-balance src-dst-mac

# 📡  Layer 2 — CDP & LLDP (Device Discovery)

CDP (Cisco) and LLDP (open standard) discover directly connected devices. Essential for mapping unknown topology.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show cdp neighbors | Connected devices: hostname, local port, remote port, platform |
| show cdp neighbors detail | Full detail: IP address, IOS version, capabilities |
| show cdp interface g0/1 | CDP status on a specific interface |
| show cdp entry * | All CDP entries with full detail |
| show lldp neighbors | LLDP neighbor list (any vendor) |
| show lldp neighbors detail | Full LLDP detail including IP and capabilities |
| show lldp interface g0/1 | LLDP status on a specific port |
| no cdp run | Disable CDP globally (security) |
| no cdp enable | Disable CDP on one interface (security) |
| lldp run | Enable LLDP globally |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| What is connected to this switch port? | show cdp neighbors | Instant neighbor discovery |
| Need IP of connected switch | show cdp neighbors detail | Shows management IP of neighbor |
| Mapping unknown topology | show cdp neighbors detail | Build network map without diagrams |
| Non-Cisco device connected | show lldp neighbors | LLDP works with all vendors |
| Verify correct cable patching | show cdp neighbors | Confirms expected device on expected port |
| Security: prevent topology leakage | no cdp enable on untrusted ports | Never run CDP toward users/internet |

## 🔄 Troubleshooting Workflow

1.  Run show cdp neighbors detail first when investigating any unknown connection

2.  Use CDP to verify correct cabling after installs

3.  Disable CDP/LLDP on all user-facing and external-facing ports

4.  Enable LLDP when working with non-Cisco switches (Aruba, Juniper, Dell)

5.  Use CDP to find IP phones: they appear as CDP neighbors

# 🔒  Layer 2 — Port Security

Port Security limits which MAC addresses can connect to an access port. Prevents unauthorized device connection.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show port-security | All ports with Port Security configured |
| show port-security interface g0/1 | Violations, max MACs, sticky MACs, current state |
| show port-security address | All secure MAC addresses (static + sticky) |
| show interfaces status ¦ include err | Quick scan for err-disabled ports |
| show errdisable recovery | Err-disabled reasons and auto-recovery settings |
| show logging ¦ include SECURITY | Port Security violation events |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Port suddenly went down after device connected | show interfaces status ¦ include err | Port Security violation → err-disabled |
| User cannot connect after moving device | show port-security interface g0/1 | Max MACs exceeded or wrong sticky MAC |
| Check if port is err-disabled | show errdisable recovery | See reason and if auto-recovery enabled |
| How many MACs learned on port? | show port-security address | View sticky/static secured MACs |
| Clear violation and re-enable port | shutdown → no shutdown | After removing unauthorized device |
| Check all security violation events | show logging ¦ include SECURITY | Find which MACs violated |

## 🔄 Troubleshooting Workflow

1.  Identify err-disabled port → show interfaces status | include err

2.  Check violation reason → show port-security interface g0/1

3.  Remove unauthorized device

4.  Clear sticky MACs if needed: clear port-security sticky interface g0/1

5.  Re-enable: interface g0/1 → shutdown → no shutdown

6.  Verify port recovered → show interfaces status

## ✅ Best Practices

- Use maximum 1 MAC on access ports for workstations

- Use violation mode restrict (log only) for initial deployment, not shutdown

- Enable err-disable autorecovery: errdisable recovery cause psecure-violation

- Document all expected MACs for sticky port security

- Regularly audit show port-security for unexpected violations

# 🌐  Layer 3 — IPv4 Addressing & Subnetting

IPv4 is the core of Layer 3. Know your subnets cold — it's the foundation of every routing decision.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip interface brief | All interfaces: IP, status (up/up, up/down, admin down) |
| show ip interface g0/0 | Detailed IP config, helper addresses, ACLs on interface |
| show running-config interface g0/0 | Exact interface config including IP |
| show version | IOS version, uptime, interfaces count |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| ipconfig /all | IP address, subnet mask, gateway, DNS, MAC — all at once |
| ipconfig /release | Release current DHCP lease |
| ipconfig /renew | Request new DHCP lease |
| ipconfig /flushdns | Clear DNS cache |
| Test-NetConnection -ComputerName 8.8.8.8 -Port 443 | Test TCP connectivity to host:port |
| Get-NetIPAddress | PowerShell: all IP addresses on all adapters |
| Get-NetIPConfiguration | IP, gateway, DNS per interface |
| netsh interface ip show config | Legacy: IP config including DNS |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| No IP address on Windows PC | ipconfig /all | Check if DHCP assigned or static config |
| Wrong subnet mask on device | ipconfig /all / show ip interface brief | Subnet mismatch = can't reach gateway |
| DHCP not giving address | ipconfig /release → /renew | Force fresh DHCP request |
| Interface shows up/down | show ip interface brief | Physical connected but no IP or no carrier |
| Interface shows admin down | show running-config interface g0/0 | Interface has 'shutdown' command |
| Device in wrong subnet | ipconfig /all + check gateway | Wrong IP or wrong default gateway set |
| DNS not resolving | ipconfig /flushdns → nslookup google.com | Stale DNS cache or wrong DNS server |

## 🔄 Troubleshooting Workflow

1.  Check client IP and gateway → ipconfig /all

2.  Verify interface is up/up → show ip interface brief

3.  If admin down → no shutdown on the interface

4.  Verify IP and mask are correct for the subnet

5.  Check gateway IP is reachable → ping <gateway>

6.  If DHCP issue → release/renew or check DHCP server

## ✅ Best Practices

- Memorize subnetting: /24=256, /25=128, /26=64, /27=32, /28=16, /29=8, /30=4

- Always verify: IP, mask, gateway, DNS before deeper troubleshooting

- Use /30 for point-to-point router links

- Document all static IPs with purpose and owner

- Avoid overlapping subnets — plan IP space before deploying

# 🗺️  Layer 3 — Routing (Static Routes & Routing Table)

The routing table decides where every packet goes. No route = packet dropped. Wrong route = packet lost.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip route | Full routing table: connected (C), static (S), OSPF (O), etc. |
| show ip route 10.0.0.0 | Find specific route and its source |
| show ip route summary | Count of routes per protocol |
| show ip route ¦ include 0.0.0.0 | Find default route (gateway of last resort) |
| show ip protocols | Active routing protocols, networks, timers |
| show ip static route | All configured static routes |
| ping 10.0.0.1 source g0/0 | Ping from a specific interface (OSPF testing) |
| traceroute 8.8.8.8 | Trace path hop by hop to destination |
| show ip interface brief | Verify all interfaces that should have routes are up |
| show running-config ¦ include ip route | All static routes in config |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| tracert 8.8.8.8 | Trace path to destination, find where it breaks |
| route print | Windows routing table |
| Test-NetConnection 8.8.8.8 -TraceRoute | PowerShell traceroute |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Ping to remote network fails | show ip route <dest> | No route = packet dropped at this router |
| Default route missing | show ip route ¦ include 0.0.0.0 | No internet = no gateway of last resort |
| Intermittent connectivity to remote site | show ip route / traceroute | Route flapping or suboptimal path |
| Traffic going wrong way | traceroute <dest> | Wrong route in table — check metrics |
| Route exists but ping fails | ping source <int> <dest> | Wrong source interface or ACL blocking |
| Static route not in table | show running-config ¦ include ip route | Check interface is up — connected route must exist |
| Routing loop suspected | traceroute — TTL expiry loop | Packets bouncing between routers |
| OSPF route not appearing | show ip protocols / show ip ospf neighbor | Neighbor not formed or network not advertised |

## 🔄 Troubleshooting Workflow

1.  Check routing table → show ip route

2.  Verify default route exists → show ip route | include 0.0.0.0

3.  If route missing → check if interface is up (connected route needed first)

4.  Test reachability with source → ping <dest> source <interface>

5.  Trace path → traceroute <dest>

6.  If route exists but fails → check ACL or NAT

## ✅ Best Practices

- Always have a default route: ip route 0.0.0.0 0.0.0.0 <next-hop>

- Use floating static routes as backup: higher AD (e.g., 5 for primary, 10 for backup)

- Verify connected routes exist before adding static routes

- Use 'show ip route <exact-subnet>' to debug specific destinations

- Document all static routes with purpose and owner

# 🔄  Layer 3 — OSPF (Open Shortest Path First)

OSPF is the most common interior gateway protocol (IGP). It automatically discovers routes and converges after topology changes.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip ospf neighbor | Neighbor state: must be FULL for working adjacency |
| show ip ospf neighbor detail | DR/BDR election, dead timer, interface |
| show ip ospf | Process ID, router ID, area info, SPF runs |
| show ip ospf interface | OSPF-enabled interfaces, area, cost, hello/dead timers |
| show ip ospf interface brief | Quick list of OSPF interfaces and their state |
| show ip ospf database | LSDB — all LSAs in the topology database |
| show ip route ospf | Only OSPF-learned routes in routing table |
| show ip protocols | OSPF process, networks advertised, redistribution |
| debug ip ospf events | Live OSPF hello/adjacency events (use carefully) |
| ping <neighbor-ip> source <interface> | Test L3 reachability between OSPF neighbors |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| OSPF neighbor not forming | show ip ospf neighbor | State stuck in EXSTART/EXCHANGE/2WAY = mismatch |
| Neighbor stuck in INIT state | show ip ospf interface | Hello received but not bidirectional |
| OSPF neighbor drops then comes back | show ip ospf neighbor / show logging | MTU mismatch or timer mismatch |
| Route from OSPF not in table | show ip route ospf | Neighbor not FULL or not in same area |
| Wrong DR elected | show ip ospf neighbor detail | Priority mismatch — set ip ospf priority |
| OSPF neighbors not forming on serial link | show ip ospf interface | Network type mismatch: point-to-point vs broadcast |
| Area mismatch | show ip ospf interface | Both sides must be in the same area |
| Timer mismatch | show ip ospf interface | Hello/dead timers must match on both sides |

## 🔄 Troubleshooting Workflow

1.  Check neighbor state → show ip ospf neighbor (must be FULL)

2.  If stuck in EXSTART → MTU mismatch (ip mtu must match or use ip ospf mtu-ignore)

3.  If neighbor shows INIT → one-way hello — check ACL blocking OSPF multicast 224.0.0.5

4.  Verify timers match → show ip ospf interface (hello 10 / dead 40 is default)

5.  Verify area matches on both interfaces

6.  Verify network type matches (show ip ospf interface)

7.  Check route appears after adjacency → show ip route ospf

## ✅ Best Practices

- OSPF neighbor must reach FULL state to exchange routes

- Common mismatches that break adjacency: area, hello/dead timers, MTU, authentication, stub flag

- Use router-id explicitly: router-id 1.1.1.1 (avoid router-id changing on reboot)

- Enable passive-interface on all non-OSPF-facing interfaces

- Use point-to-point network type on serial/tunnel links

- Avoid redistribution without filters — it can cause routing loops

# 📦  Layer 3 — DHCP & DHCP Relay (IP Helper)

DHCP dynamically assigns IP addresses. DHCP Relay (ip helper-address) forwards DHCP broadcasts across subnets.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip dhcp pool | DHCP pool name, network, options configured |
| show ip dhcp binding | All active leases: IP, MAC, expiry |
| show ip dhcp conflict | IPs that caused conflicts (duplicate detection) |
| show ip dhcp server statistics | Discover/offer/request/ack counts — diagnose drops |
| show running-config ¦ include helper | Find ip helper-address config on all interfaces |
| show ip interface g0/1 ¦ include Helper | Helper address on specific interface |
| debug ip dhcp server events | Live DHCP request/response events |
| clear ip dhcp conflict * | Clear all DHCP conflicts to allow re-assignment |
| clear ip dhcp binding * | Clear all leases — force clients to renew |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| ipconfig /release → ipconfig /renew | Force DHCP renewal on Windows client |
| ipconfig /all | Check current IP, DHCP server, lease expiry |
| Test-NetConnection <DHCP-server-IP> -Port 67 | Test if DHCP server is reachable |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Client gets 169.254.x.x (APIPA) | ipconfig /release → /renew → check DHCP server | DHCP server unreachable or pool exhausted |
| DHCP not working across VLANs | show running-config ¦ include helper | ip helper-address missing on SVI/interface |
| Pool exhausted | show ip dhcp binding | All IPs allocated — increase pool or shorten lease |
| Duplicate IP conflicts | show ip dhcp conflict | Conflict detected — clear ip dhcp conflict * |
| Client gets wrong subnet/gateway | show ip dhcp pool | Wrong default-router or wrong pool subnet |
| DHCP works locally but not remotely | show ip interface ¦ include Helper | ip helper-address missing on remote VLAN interface |
| DHCP lease expired but client kept IP | clear ip dhcp binding <IP> | Force lease release for specific IP |

## 🔄 Troubleshooting Workflow

1.  Client gets 169.254.x.x → check DHCP server reachability

2.  Verify ip helper-address on SVI → show running-config | include helper

3.  Check pool has free IPs → show ip dhcp binding (count entries vs pool size)

4.  Check for conflicts → show ip dhcp conflict → clear ip dhcp conflict *

5.  Force client renewal → ipconfig /release → /renew

6.  If pool exhausted → expand pool range or reduce lease time

## ✅ Best Practices

- Always configure ip helper-address on each SVI/interface that has clients in a different subnet from DHCP server

- Reserve IPs for printers/servers: ip dhcp excluded-address 192.168.1.1 192.168.1.20

- Use short DHCP lease for guest/wireless (1-4 hours)

- Monitor pool utilization: show ip dhcp binding | count

- Clear conflicts regularly: clear ip dhcp conflict *

- Always configure default-router and dns-server in every DHCP pool

# 🔁  Layer 3 — NAT (Network Address Translation)

NAT translates private IPs to public IPs. Without it, private addresses cannot reach the internet.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip nat translations | Active NAT sessions: inside local ↔ inside global ↔ outside |
| show ip nat translations verbose | Full detail including timeouts per translation |
| show ip nat statistics | Hit/miss counts, active translations, interfaces |
| show running-config ¦ include nat¦overload | NAT config: ip nat inside/outside, pool or overload |
| show ip interface g0/0 ¦ include NAT | Verify nat inside/outside direction on interface |
| debug ip nat | Live NAT translation events (use carefully — verbose) |
| clear ip nat translation * | Clear all active NAT translations (reconnects sessions) |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Internet not working for internal users | show ip nat statistics | NAT miss = traffic not being translated |
| Specific host cannot reach internet | show ip nat translations ¦ include <IP> | No translation = ACL excludes host from NAT |
| NAT translation table full | show ip nat statistics | Too many sessions — check for attack or increase timeout |
| VPN traffic being NATed incorrectly | show running-config ¦ include nat | VPN traffic should be excluded from NAT via ACL |
| Verify which interface is inside/outside | show ip interface ¦ include NAT | Wrong direction breaks all NAT |
| External host cannot reach internal server (static NAT) | show ip nat translations ¦ include static | Static NAT entry missing or wrong |
| Clear stuck NAT sessions | clear ip nat translation * | Force fresh translations after config change |

## 🔄 Troubleshooting Workflow

1.  Verify inside/outside interface designation → show ip interface | include NAT

2.  Check if translations are being created → show ip nat translations

3.  If no translations → check ACL permits the host

4.  If translations exist but no internet → check default route

5.  Verify NAT config → show running-config | include nat

6.  Clear stale entries after changes → clear ip nat translation *

## ✅ Best Practices

- Always mark correct interfaces: ip nat inside (LAN) and ip nat outside (WAN)

- Use overload (PAT) for multiple clients: ip nat inside source list <ACL> interface <WAN> overload

- Exclude VPN traffic from NAT using route-map or extended ACL

- Use static NAT for servers that must be reachable from outside

- Monitor show ip nat statistics daily in production — high misses = config problem

- Clear NAT table after any config change: clear ip nat translation *

# 📶  Layer 3 — ICMP, Ping & Traceroute

Ping tests reachability. Traceroute finds where packets stop. Master these two before any other tool.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| ping <IP> | Basic reachability test — 5 packets |
| ping <IP> repeat 100 | Extended ping — more reliable result |
| ping <IP> source <interface> | Ping from a specific interface (critical for OSPF/routing tests) |
| ping <IP> source <interface> repeat 1000 size 1500 | Test with full MTU frames |
| traceroute <IP> | Hop-by-hop path to destination — find where it breaks |
| traceroute <IP> source <interface> | Traceroute from specific interface |
| show ip interface brief ¦ include up | Confirm source interface is up before pinging |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| ping <IP> | Test reachability from Windows |
| ping <IP> -t | Continuous ping (stop with Ctrl+C) |
| ping <IP> -l 1400 | Ping with specific packet size (MTU testing) |
| ping <IP> -n 100 | Send 100 pings for reliability |
| tracert <IP> | Trace route with DNS resolution |
| tracert -d <IP> | Trace route without DNS resolution (faster) |
| pathping <IP> | Combined ping + traceroute with loss statistics |
| Test-NetConnection <IP> | PowerShell: ping + port test |
| Test-NetConnection <IP> -Port 443 | Test if TCP port is open |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Ping fails — basic test | ping <gateway-IP> | Start at gateway — isolate local vs remote |
| Ping works but app fails | Test-NetConnection <IP> -Port 443 | IP reachable but port blocked (L4/firewall) |
| Need to find where route breaks | tracert -d <IP> | Find the last responding hop |
| Ping inconsistent — some drops | ping <IP> -t -l 1400 | MTU issue or unstable link |
| Ping router's own interface | ping <IP> source g0/0 | Test from router perspective, not PC |
| Test MTU along path | ping <IP> -l 1400 -f (do-not-fragment) | Find MTU bottleneck in path |
| Test packet loss over time | pathping <IP> | Per-hop loss statistics |

## 🔄 Troubleshooting Workflow

1.  Always ping the gateway first — if gateway fails, stop there

2.  ping <IP> → if fail → ping gateway → if ok → routing issue beyond router

3.  tracert -d → find last responding hop → that's where route breaks

4.  On router: always use 'ping source' to test from correct interface

## ✅ Best Practices

- Always ping bottom-up: gateway → next hop → destination

- Use -d flag in tracert to skip DNS and speed up results

- Use ping source on Cisco to control which interface traffic exits

- Large ping (-l 1472) tests MTU — if this fails and small ping works = MTU issue

- pathping is the best single-command connectivity diagnostic on Windows

- * in tracert = firewall or ICMP rate-limiting, NOT necessarily packet loss

# 🚚  Layer 4 — Transport (TCP & UDP)

TCP = reliable, connection-oriented. UDP = fast, connectionless. Layer 4 issues = port blocked, firewall, or service not listening.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip access-lists | View ACLs — may be blocking TCP/UDP ports |
| show ip access-lists ¦ include deny | Find deny entries that may block traffic |
| show interfaces g0/0 ¦ include rate | Check for interface congestion causing drops |
| debug ip packet detail | Live packet inspection — use only in lab/emergency |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| netstat -an | All active TCP/UDP connections and listening ports |
| netstat -b | Show which process owns each connection (run as admin) |
| netstat -an ¦ findstr :443 | Check if specific port is listening or connected |
| Test-NetConnection <IP> -Port 80 | Test TCP port connectivity |
| Test-NetConnection <IP> -Port 22 -InformationLevel Detailed | Detailed TCP test |
| Get-NetTCPConnection | PowerShell: all TCP connections |
| Get-NetUDPEndpoint | PowerShell: all UDP listeners |
| telnet <IP> <port> | Legacy port test (enable feature first in Windows) |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| App fails but ping works | Test-NetConnection <IP> -Port <N> | Port blocked by firewall or service not running |
| Port is open but connection refused | netstat -an ¦ findstr :<port> | Service not listening on that port |
| Random TCP drops | netstat -an — check TIME_WAIT count | Too many connections — server-side tuning needed |
| Which app owns a port? | netstat -b (admin) | See process name per connection |
| SSH not working to switch | show ip access-lists | ACL blocking port 22 |
| Check if VPN port is reachable | Test-NetConnection <VPN-IP> -Port 4500 | UDP 500/4500 for IPsec |

## 🔄 Troubleshooting Workflow

1.  Ping passes → service fails → it's L4 or L7 (not routing)

2.  Test-NetConnection <IP> -Port <N> → TcpTestSucceeded: True = port open

3.  If port closed → check firewall on server and network ACLs

4.  netstat -an → verify service is listening on correct port and IP

5.  netstat -b → identify which process owns the port

## ✅ Best Practices

- Use Test-NetConnection before blaming the network — it proves port reachability

- TCP 3-way handshake: SYN → SYN-ACK → ACK — if stuck in SYN_SENT = port blocked or server issue

- UDP has no handshake — use application-level tests for UDP services

- Document all required ports for each application in your environment

- Check both Windows Firewall AND network ACLs when a port is unreachable

# 🔍  Application Layer — DNS Troubleshooting

DNS resolves hostnames to IPs. DNS failure = everything appears broken even when the network is fine.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip dns view | DNS view configuration on router |
| show hosts | Static and cached hostname entries on router |
| ip name-server 8.8.8.8 | Configure DNS server on router |
| ping cisco.com | Test DNS resolution from router |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| nslookup google.com | Test DNS resolution — shows which DNS server is used |
| nslookup google.com 8.8.8.8 | Test resolution against a specific DNS server |
| Resolve-DnsName google.com | PowerShell DNS query |
| Resolve-DnsName google.com -Server 8.8.8.8 | Query specific DNS server via PowerShell |
| ipconfig /displaydns | Show cached DNS entries |
| ipconfig /flushdns | Clear DNS cache |
| ipconfig /all | Check configured DNS server IP |
| Test-NetConnection -ComputerName google.com | DNS + connectivity test in one command |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Browser says 'server not found' | nslookup google.com | DNS resolution failing — check DNS server |
| Ping by name fails but IP works | nslookup <hostname> | DNS issue, not network issue |
| DNS slow to resolve | nslookup — check response time | Wrong or overloaded DNS server |
| Internal hostname not resolving | nslookup <hostname> <internal-DNS-IP> | DNS server may not have internal zone |
| DNS after network change | ipconfig /flushdns | Stale cache with old IP |
| Check which DNS server is configured | ipconfig /all | Look for 'DNS Servers' line |
| Test DNS from router | ping cisco.com (on router) | If fails, router DNS config is wrong |

## 🔄 Troubleshooting Workflow

1.  If name fails but IP works → DNS issue, not network → nslookup first

2.  nslookup google.com → note the DNS server IP being used

3.  Try nslookup google.com 8.8.8.8 → if works, local DNS is the problem

4.  ipconfig /flushdns → test again

5.  Check DNS server IP → ipconfig /all → look for DNS Servers line

6.  Verify DNS server is reachable → Test-NetConnection <DNS-IP> -Port 53

## ✅ Best Practices

- Always configure 2 DNS servers (primary + secondary) — never just one

- Use 8.8.8.8 as secondary for external resolution testing

- ipconfig /flushdns is your first DNS fix — always start there

- nslookup <internal-host> proves internal DNS — nslookup <external> proves internet DNS

- DNS over port 53 UDP/TCP — check both if Test-NetConnection fails

- In corporate env: internal DNS should forward to ISP/8.8.8.8 for external names

# 🔐  Application Layer — SSH & Remote Access Troubleshooting

SSH is the primary secure management protocol. Misconfigured SSH = locked out of device.

## 🔵 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip ssh | SSH version, authentication timeout, retries |
| show ssh | Active SSH sessions |
| show line vty 0 4 | VTY line config (transport, timeout, ACL) |
| show running-config ¦ include ssh¦line vty¦access-class | Full SSH config snapshot |
| show ip access-lists <name> | Check ACL applied to VTY lines |
| show users | Active connected users and their lines |
| show crypto key mypubkey rsa | Verify RSA key exists for SSH |

## 🟢 Windows / PowerShell Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| Test-NetConnection <device-IP> -Port 22 | Test if SSH port is reachable |
| ssh -v admin@192.168.1.1 | Verbose SSH connection — shows where it fails |
| netstat -an ¦ findstr :22 | Check if SSH port is listening on server |

## ⚡ Flashcards — Problem → Command

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| Cannot SSH to switch/router | show ip ssh | SSH may be version 1 or not enabled |
| SSH times out — port unreachable | Test-NetConnection <IP> -Port 22 | ACL blocking or VTY not configured |
| SSH refused — key error | show crypto key mypubkey rsa | RSA key not generated — crypto key generate rsa |
| SSH works but disconnects quickly | show line vty 0 4 ¦ include timeout | exec-timeout too short |
| Too many SSH sessions | show users / show ssh | All VTY lines occupied — disconnect idle sessions |
| ACL blocking SSH from my subnet | show ip access-lists <name> | Add permit statement for your IP |

## 🔄 Troubleshooting Workflow

1.  Test port 22 → Test-NetConnection <IP> -Port 22

2.  Check SSH is enabled → show ip ssh

3.  Verify RSA key → show crypto key mypubkey rsa

4.  Check VTY config → show line vty 0 4

5.  Check ACL on VTY → show running-config | include access-class

6.  Check if all lines busy → show users

## ✅ Best Practices

- Minimum SSH v2 config: ip ssh version 2, crypto key generate rsa modulus 2048

- Always apply ACL to VTY lines: access-class <ACL> in

- Set exec-timeout: exec-timeout 10 0 (10 minutes)

- Set login local: transport input ssh (not telnet!)

- Always have console access as backup if SSH breaks

- SSH requires: hostname set, domain name set, RSA key, local user

# ⚡ Master Quick Reference — Most Used Commands

Keep this page open during live troubleshooting. These commands solve 80% of real incidents.

## 🔴 TOP 20 Cisco IOS Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| show ip interface brief | All interfaces: IP, up/up status — START HERE |
| show interfaces status | All ports: connected/notconnect/err-disabled |
| show vlan brief | VLAN membership, all ports |
| show interfaces trunk | Active trunks + allowed VLANs |
| show ip route | Full routing table — find next hop |
| show ip ospf neighbor | OSPF neighbor state — must be FULL |
| show ip dhcp binding | Active DHCP leases |
| show mac address-table | All learned MACs on switch |
| show arp / show ip arp | IP↔MAC mappings |
| show spanning-tree | Root bridge, port roles, STP state |
| show interfaces g0/1 | Errors, speed, duplex, CRC for one port |
| show interfaces counters errors | CRC, runts, giants — physical layer health |
| show port-security interface g0/1 | Port Security violations and state |
| show errdisable recovery | Err-disabled reason and recovery state |
| show cdp neighbors detail | Identify connected devices and their IPs |
| show etherchannel summary | Port-channel bundle state |
| show ip nat translations | Active NAT sessions |
| show running-config interface g0/1 | Exact config of one interface |
| show logging ¦ tail | Last syslog events |
| show processes cpu sorted | CPU utilization — detect broadcast storm |

## 🟢 TOP 15 Windows Commands

| COMMAND | WHAT IT SHOWS |
| --- | --- |
| ipconfig /all | IP, mask, gateway, DNS, MAC — everything |
| ping <IP> -t | Continuous ping — watch for drops |
| tracert -d <IP> | Trace path without DNS — faster |
| pathping <IP> | Per-hop loss + latency statistics |
| arp -a | Local ARP table (IP↔MAC) |
| arp -d * | Flush ARP cache |
| nslookup google.com | Test DNS resolution |
| ipconfig /flushdns | Clear DNS cache |
| ipconfig /release → /renew | Force DHCP renewal |
| netstat -an | All TCP/UDP connections and listeners |
| netstat -b | Show process per connection (admin) |
| Test-NetConnection <IP> -Port 443 | Test TCP port reachability |
| route print | Windows routing table |
| Get-NetAdapter | NIC status, speed, MAC |
| netstat -an ¦ findstr LISTEN | All listening ports — what services are running |

## 🧠 Troubleshooting Decision Tree

| PROBLEM / SYMPTOM | COMMAND | WHY |
| --- | --- | --- |
| No network at all — new device | show interfaces status → show vlan brief | L1/L2 first |
| User gets wrong IP | show interfaces g0/1 switchport | Wrong VLAN → wrong DHCP scope |
| Ping to gateway fails | ipconfig /all → check gateway IP → ping | Default gateway wrong or interface down |
| Ping gateway OK, internet fails | show ip route ¦ include 0.0.0.0 | Default route or NAT missing |
| Same subnet, cannot reach device | arp -a → check MAC → show mac address-table | ARP/L2 issue |
| DNS works, app still fails | Test-NetConnection <IP> -Port <N> | Firewall or service issue — L4/L7 |
| Intermittent drops everywhere | show spanning-tree / show processes cpu | Broadcast storm or STP loop |
| Port went err-disabled | show errdisable recovery → show port-security | Port Security or BPDU Guard |
| OSPF route missing | show ip ospf neighbor → must be FULL | Adjacency broken — check timers/area/MTU |
| After device replacement | arp -d * / clear arp-cache | Stale ARP entries on all clients |